线定义：

红色： 3.3V

黑色：SDA

黄色：SCL

绿色：GND

算法：

- NA2302器件地址为：0x00

- 压力0x06 温度地址0x09

量程计算

P（压力）=【Value（采集值）/8388608-0.2】/0.7 *Pr（压力范围）

举例：

1MPA压力传感器，IIC得到Value=4194304，于是

P（压力）=【4194304/8388608-0.2】/0.7 *（1MPA-0） =0.428MPA

程序：

Iic.h

#ifndef __IIC_H

#define __IIC_H

#include "sys.h"

#include "delay.h"

//IO操作函数

#define IIC_SCL    PAout(5) //SCL

#define IIC_SDA    PAout(4) //SDA

#define READ_SDA   PAin(4)  //输入SDA

//IO方向设置 CRL(Pin0~7)

#define SDA_IN()   {SDA_IN_Status();}

#define SDA_OUT()  {SDA_OUT_Status();}

void IIC_Initializes(void);

//IIC所有操作函数

void IIC_Init(void);                							//初始化IIC的IO口

void IIC_Start(unsigned short num);								//发送IIC开始信号

void IIC_Stop(unsigned short num);	  						//发送IIC停止信号

unsigned char IIC_Wait_Ack(unsigned short num); 	//IIC等待ACK信号

void IIC_Ack(unsigned short num);									//IIC发送ACK信号

void IIC_NAck(unsigned short num);								//IIC不发送ACK信号

void IIC_Send_Byte(unsigned short num, unsigned char T_Data);					//IIC发送一个字节

unsigned char IIC_Read_Byte(unsigned short num);										  //IIC读取一个字节

unsigned char IIC_Write_Len_Byte(unsigned short num, unsigned char dev_addr,unsigned char reg_addr, unsigned char data_len, unsigned char* wData);

unsigned char IIC_Read_Len_Byte(unsigned short num, unsigned char dev_addr,unsigned char reg_addr, unsigned char data_len, unsigned char* rData);

void Get_CalData_IIC (uint8_t num, uint8_t Mode, uint8_t OutFormat);

void NSA2302_CalData(void);

void SDA_OUT_Status(void);

void SDA_IN_Status(void);

#endif

//////////////////////////////////////////////////////////////////////////////////////////////////////

Iic.c

#include "usart.h"

#include "iic.h"

unsigned short Num = 10;

extern uint8_t Cal_Pdata[3];//读取校准后压力数组

extern uint8_t Cal_Tdata[3];//读取校准后温度数组

extern uint8_t Cal_data[6];//读取校准后数组

extern float Cal_PData;//读取校准后压力数组

extern float Cal_TData;//读取校准后温度数组

extern uint8_t ID_Num_W[6];

extern uint8_t ID_Num_R[6];

extern uint8_t Drdy;

////////////////实际测试，最快num=4，否则IO反转速率不够//////////////

void IIC_Initializes(void)                      //IIC初始化

{

IIC_Init();

}

//初始化IIC

void IIC_Init(void)

{

GPIO_InitTypeDef GPIO_InitStructure;

RCC_APB2PeriphClockCmd(	RCC_APB2Periph_GPIOA, ENABLE );

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_4|GPIO_Pin_5;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_OD ;   	//开漏输出

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

IIC_SCL=1;

IIC_SDA=1;

}

void SDA_OUT_Status(void)

{

GPIO_InitTypeDef GPIO_InitStructure;

RCC_APB2PeriphClockCmd(	RCC_APB2Periph_GPIOA, ENABLE );

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_4;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_OD ;      //开漏输出

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

}

void SDA_IN_Status(void)

{

GPIO_InitTypeDef GPIO_InitStructure;

RCC_APB2PeriphClockCmd(	RCC_APB2Periph_GPIOA, ENABLE );

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_4;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING ; //浮空输入

GPIO_Init(GPIOA, &GPIO_InitStructure);

}

//产生IIC起始信号

void IIC_Start(unsigned short num)

{

SDA_OUT();     //SDA线输出

IIC_SDA=1;

IIC_SCL=1;

delay_us(num);

IIC_SDA=0;		 //Start标志

delay_us(num);

IIC_SCL=0;		 //钳住I2C总线，准备发送或接收数据

}

//产生IIC停止信号

void IIC_Stop(unsigned short num)

{

IIC_SCL=0;

SDA_OUT();		 //SDA线输出

IIC_SDA=0;		 //STOP标志

delay_us(num);

IIC_SCL=1;

delay_us(num);

IIC_SDA=1;		 //发送结束信号

delay_us(num);

}

//等待应答信号到来

//返回值：1，接收应答失败

//        0，接收应答成功

unsigned char IIC_Wait_Ack(unsigned short num)

{

unsigned char ucErrTime=0;

delay_us(num/4);IIC_SCL=1;

SDA_IN();      										//SDA设置为输入

while(READ_SDA)

{

ucErrTime++;

if(ucErrTime>250)

{

IIC_Stop(num);

return 0xff;

}

}

IIC_SCL=0;

delay_us(num/2);

IIC_SDA=1;												//时钟输出0

return 0x00;

}

//产生ACK应答

void IIC_Ack(unsigned short num)

{

IIC_SCL=0;

SDA_OUT();

IIC_SDA=0;

delay_us(num/2);

IIC_SCL=1;

delay_us(num/2);

IIC_SCL=0;

}

//不产生ACK应答

void IIC_NAck(unsigned short num)

{

IIC_SCL=0;

SDA_OUT();

IIC_SDA=1;

delay_us(num/2);

IIC_SCL=1;

delay_us(num/2);

IIC_SCL=0;

}

//IIC发送一个字节

//返回从机有无应答

//1，有应答

//0，无应答

void IIC_Send_Byte(unsigned short num, unsigned char T_Data)

{

unsigned char t;

SDA_OUT();

IIC_SCL=0;									//拉低时钟开始数据传输

for(t=0;t<8;t++)

{

IIC_SDA=(T_Data&0x80)>>7;

T_Data<<=1;

delay_us(num/4);

IIC_SCL=1;

delay_us(num/2);

IIC_SCL=0;

delay_us(num/4);

}

}

//读1个字节，ack=1时，发送ACK，ack=0，发送nACK

unsigned char IIC_Read_Byte(unsigned short num)

{

unsigned char i,receive=0;

SDA_IN();											//SDA设置为输入

for(i=0;i<8;i++ )

{

IIC_SCL=0;

delay_us(num/2);

IIC_SCL=1;

receive<<=1;

if(READ_SDA)receive++;

delay_us(num/2);

}

return receive;

}

unsigned char IIC_Read_Len_Byte(unsigned short num, unsigned char dev_addr,unsigned char reg_addr, unsigned char data_len, unsigned char* rData)

{

unsigned char i = 0;

unsigned char dev_addr_r = dev_addr | 0x01;

unsigned char dev_addr_w = dev_addr & 0xFE; //防止读写IIC地址出错，写统一地址即可

IIC_Start(num);  														//IIC开始

IIC_Send_Byte(num, dev_addr_w);							//先发设备地址

if(IIC_Wait_Ack(num) == 0xff)  							//没有等到Ack返回则返回0xFF

{ return 0xff; }

IIC_Send_Byte(num, reg_addr);  							//开始发送寄存器地址

if(IIC_Wait_Ack(num) == 0xff)

{ return 0xff; }

IIC_Start(num);  														//IIC开始

IIC_Send_Byte(num, dev_addr_r);							//开始发读命令

if(IIC_Wait_Ack(num) == 0xff)

{ return 0xff; }

for(i=0; i<data_len-1; i++)

{

rData[i] = IIC_Read_Byte(num);

IIC_Ack(num); //发送ACK

}

rData[data_len -1] = IIC_Read_Byte(num);

IIC_NAck(num);//发送nACK

IIC_Stop(num);															//产生一个停止条件

return 0x00;

}

unsigned char IIC_Write_Len_Byte(unsigned short num, unsigned char dev_addr,unsigned char reg_addr, unsigned char data_len, unsigned char* wData)

{

unsigned char i = 0;

unsigned char dev_addr_w = dev_addr & 0xFE; //防止读写IIC地址出错，写统一地址即可

IIC_Start(num);  														//IIC开始

IIC_Send_Byte(num, dev_addr_w);							//先发设备地址

if(IIC_Wait_Ack(num) == 0xff)  							//没有等到Ack返回则返回0xFF

{ return 0xff; }

IIC_Send_Byte(num, reg_addr);  							//开始发送寄存器地址

if(IIC_Wait_Ack(num) == 0xff)

{ return 0xff; }

if(data_len == 1)                  				 	//单次写入

{

IIC_Send_Byte(num, wData[data_len-1]);

if(IIC_Wait_Ack(num) == 0xff)

{ return 0xff; }

}

else

{

for(i=0; i<data_len; i++)       					  //连续写入

{

IIC_Send_Byte(num, wData[i]);

if(IIC_Wait_Ack(num) == 0xff)

{ return 0xff; }

}

}

IIC_Stop(num);

return 0x00;

}

void NSA2302_CalData(void)

{

while((Drdy & 0x01) == 0x01 )

{

IIC_Read_Len_Byte(4 ,0xFE, 0x02, 1, &Drdy);

delay_us(4);

}

IIC_Read_Len_Byte(4, 0xFE, 0x06, 5, ID_Num_R);

Cal_PData = ID_Num_R[0] *65535 + ID_Num_R[1] *256 + ID_Num_R[2];

Cal_TData = ID_Num_R[3] *65535 + ID_Num_R[4] *256;

if(Cal_PData > 8388608) {	Cal_PData = (Cal_PData - 16777216)/ 8388608; }

else { Cal_PData = Cal_PData / 8388608; }

if(Cal_TData > 8388608) {	Cal_TData = (Cal_TData - 16777216)/ 65535; }

else { Cal_TData = Cal_TData / 65535; }

printf("0x06：%d  0x07：%d  0x08：%d  \r\n ", ID_Num_R[0], ID_Num_R[1], ID_Num_R[2]);

printf("0x09：%d  0x0A：%d  \r\n ", ID_Num_R[3], ID_Num_R[4]);

printf("压力数字量：%f 温度值：%f \r\n ", Cal_PData, Cal_TData);

printf("第一个数据：0x%X  第二个数据：0x%X  第三个数据：0x%X  \r\n ", ID_Num_R[0], ID_Num_R[1], ID_Num_R[2]);

printf("第四个数据：0x%X  第五个数据：0x%X  第六个数据：0x%X  \r\n ", ID_Num_R[3], ID_Num_R[4], ID_Num_R[5]);

}

void Get_REG_Value_IIC (uint8_t research_register)

{

uint8_t receive_data =0;

IIC_Read_Len_Byte(10, 0xDA, research_register, 1, &receive_data);

printf("寄存器 %#x 值为：%#x\n \r\n ", research_register , receive_data);

delay_ms(1000);

}